DESIGN REPORT

Rocker-Bogie Suspension and Drive System

ROVER "Target Fidelity" — a low-cost all-terrain autonomous rover

Chassis body: 600 mm (W) × 1200 mm (L)

Target speed: 150 mm/s   ·   Obstacle clearance: 1.5 × wheel diameter (300 mm)

A quick note on how to read this. You don’t need to track down the source papers to follow along. Whenever a number comes from someone else’s work, it’s cited in APA style (author, year) right where it’s used, and there’s a short plain-language explanation nearby so you know what that work actually says. All dimensions here match the current Fusion 360 model.

Abstract

This report works through the design and sizing of the chassis and passive suspension for ROVER "Target Fidelity," a six-wheeled, low-cost rover meant to drive itself over rough ground (dirt, gravel, grass). The suspension is a rocker-bogie mechanism, borrowed from planetary rovers and scaled down to a 600 × 1200 mm body. The design targets a cruising speed of 150 mm/s and the ability to clear vertical steps up to 1.5 times the wheel diameter (300 mm). The arms are round aluminum tube joined by 3D-printed PETG fittings, the differential axle is a Ø12 mm steel rod, and the four corner wheels steer using hobby servos. Structural checks are done with static finite-element analysis (FEA) in the Autodesk Fusion 360 Simulation workspace. The two reference studies behind the geometry are Verma et al. (2017) and Singh and Jain (2020).

Keywords: rocker-bogie, passive suspension, all-terrain robot, low-cost rover, FEA, obstacle climbing

# 1.  Introduction

Ordinary wheeled suspensions are built for paved or semi-structured surfaces, and they tend to fall apart on ground where the soil, the rocks, and the slope keep changing. A rover collecting field data in agriculture, geology, or ecology has to cross exactly that kind of terrain, and a stiff four-wheel platform would get stuck fast. The rocker-bogie mechanism, developed at NASA’s Jet Propulsion Laboratory for the Sojourner rover in 1997 (Bickler, 1998), gets around this by spreading the rover’s weight across all six wheels through a purely passive linkage — no springs, no shock absorbers, no powered suspension.

The overall goal of the project is a rover that moves on its own across rough terrain, starting with the drive system and a sampling arm. This report covers the drive system: it lays out the full rocker-bogie geometry for the 600 × 1200 mm chassis at 150 mm/s with a 300 mm obstacle target (that’s 1.5 × the wheel diameter), and it explains how to build and check the whole thing in Fusion 360.

# 2.  Design Principles

Before the math, it helps to pin down three ideas that drive every later decision. Each one leans on earlier work, summarized in plain terms so the source papers stay optional reading.

## 2.1  Six wheels, and nothing actively driven in the suspension

The rover runs six wheels in a 3-3 layout — three per side. This keeps at least four wheels on the ground at any moment, which spreads out the contact pressure and makes the rover hard to tip. The whole suspension is passive: the geometry of the arms does the work of adapting to the terrain, with no springs and no suspension motors to fail.

## 2.2  The speed formula and the arm lengths

The starting dimensions — how fast the wheel spins and how long the arms are — come from three fairly simple geometric relationships (Verma et al., 2017):

V = π · D · N / 60      [1]

L_arm = reach / cos θ      [2]

h_arm = reach · tan θ      [3]

Here V is linear speed (mm/s), D is wheel diameter (mm), N is rpm, θ is the arm angle from horizontal, "reach" is the arm’s horizontal span, L_arm is its true length, and h_arm is the height it adds. Worth knowing: Verma et al. (2017) used θ = 45° because their prototype climbed vertical stairs. This design uses gentler angles (10–15°) since it’s built for open terrain, not staircases — more on that in Section 6.5.

## 2.3  Where the bogie pivot goes (load sharing)

The bogie is the small rocking arm that carries two of the three wheels on each side. Where you put its pivot decides how the weight splits between those two wheels. Singh and Jain (2020) modeled this and found two useful options:

- Case 1 — offset pivot (closer to the front wheel): weight splits 0.21 / 0.39 / 0.40 across the front, middle, and rear wheels. Uneven.

- Case 2 — centered pivot (equal distance to both bogie wheels): weight splits 0.30 / 0.30 / 0.40. The front and middle wheels carry the same load.

This design uses Case 2. When the front and middle wheels carry equal load, there’s less chance of one spinning out on loose ground. Either way, the rear wheel always takes 40% of the weight (Singh & Jain, 2020). That same study also recommends elastic (rubber-like) tires for better grip on slopes, which is why the wheel here uses a soft TPU tread (Section 8).

# 3.  Requirements

| ID | Requirement | Target |
| --- | --- | --- |
| R-01 | Chassis body width (minimum) | 600 mm |
| R-02 | Chassis body length (minimum) | 1200 mm |
| R-03 | Travel speed | 150 mm/s |
| R-04 | Vertical obstacle clearance | H = 1.5 D = 300 mm |
| R-05 | Driven wheels | 6 (3 per side) |
| R-06 | Suspension type | Passive rocker-bogie |
| R-07 | Steering | Servos on the 4 corner wheels |
| R-08 | Terrain | Dirt, gravel, grass |
| R-09 | Estimated total mass | ≤ 22 kg (preliminary) |

Table 1. Design requirements and targets.

# 4.  Input Parameters

| Parameter | Symbol | Value | Why |
| --- | --- | --- | --- |
| Body width | W_b | 600 mm | Requirement R-01 |
| Body length | L_b | 1200 mm | Requirement R-02 |
| Speed | V | 150 mm/s | Requirement R-03 |
| Obstacle target | H | 300 mm | R-04: 1.5 × D |
| Wheel diameter | D | 200 mm | D ≈ W_b / 3; common ratio |
| Wheel radius | R | 100 mm | R = D / 2 |
| Total mass | m | 22 kg | Estimate with all subsystems |
| Bogie configuration | — | Case 2 | Centered pivot (§2.3) |

Table 2. Fixed input parameters.

# 5.  Geometry and Kinematics

## 5.1  Wheel speed

Solving formula [1] for N at the chosen speed and diameter:

N = 60 · V / (π · D) = 60 × 150 / (π × 200) = 14.32 rpm

So each motor needs to turn the wheel at about 14.3 rpm. JGB37-520 gearmotors (roughly 100 rpm at 12 V) handle that with plenty of room to spare, and the actual speed is set by the driver’s PWM. If more rpm turns out to be needed, the fix is an extra gear stage or a different wheel diameter.

## 5.2  Wheelbase and wheel positions

The wheelbase comes from subtracting the front and rear wheel radii from the body length:

Wheelbase = L_b − (R + R) = 1200 − (100 + 100) = 1000 mm

That places the three wheels on each side as follows, measured from the front wheel:

| Point | X position | Notes |
| --- | --- | --- |
| Front wheel (W1) | 0 mm | Reference |
| Middle wheel (W2) | 430 mm | Same load as the front (Case 2) |
| Rear wheel (W3) | 1000 mm | End of the wheelbase |
| Bogie pivot | 215 mm | Centered between W1 and W2 |
| Chassis pivot (differential) | 615 mm | Where the rocker meets the chassis |

Table 3. Longitudinal positions (one side).

## 5.3  Load sharing

Applying the Case 2 split (§2.3) to the total weight W = m·g = 22 × 9.81 = 215.8 N, the per-wheel reactions are:

R3 (rear) = 0.40 · W = 86.3 N

R1 (front) = R2 (middle) = 0.30 · W = 64.7 N

The rear wheel is the most heavily loaded, which makes the rocker arm (the part holding it) the piece to worry about. Adding a ×2 dynamic factor for the bumps and shocks of rough ground, the design load on the rocker works out to roughly 173 N. Treating that as a worst-case load concentrated on one wheel keeps the structural check on the safe side (Singh & Jain, 2020).

## 5.4  Obstacle climbing

The target is to clear 300 mm steps, which is 1.5 × D. The theoretical ceiling for a rocker-bogie is 2 × D = 400 mm, climbing in sequence with all six wheels staying in contact. So the 300 mm target sits at 75% of that ceiling, leaving a comfortable 1.33 margin against the geometric limit (Singh & Jain, 2020).

## 5.5  Arm angles

The angles are picked for open terrain rather than step-climbing. Instead of the 45° from the original stair-climbing prototype (§2.2), this design uses θ_rocker = 10° and θ_bogie = 15°, which give enough ground clearance without raising the center of gravity too much.

### 5.5.1  Bogie arm

Each side’s bogie has two equal arms (Case 2), each with a horizontal reach of 215 mm:

L_bogie = 215 / cos 15° = 222.6 mm ≈ 223 mm (center to center)

h_bogie = 215 × tan 15° = 57.6 mm (pivot height above the wheels)

Since both arms sit at 15° and form a symmetric V, the angle between the two arm axes — measured at the pivot, center to center of each socket — is:

Bogie angle (Joint B) = 180° − 15° − 15° = 150°

### 5.5.2  Rocker arm

The rocker runs from the chassis pivot (x = 615 mm) forward to the bogie pivot (x = 215 mm), and back to the rear wheel (x = 1000 mm). Each segment uses θ = 10°:

Forward segment: 395 / cos 10° = 401 mm

Rear segment: 390 / cos 10° = 396 mm

Total rocker length = 401 + 396 ≈ 800 mm

The angle between the two rocker segments (Joint A) is 180° − 10° − 10° = 160°.

## 5.6  Ground clearance

The base clearance under the chassis pivot is h_bogie + R = 57.6 + 100 = 157.6 ≈ 158 mm. But the wheels don’t bolt straight onto the tubes — they hang from mounts that drop them 90 mm lower (see §7), so the chassis rises by that same amount and the real clearance becomes about 248 mm. That’s better for clearing obstacles, at the cost of a slightly higher center of gravity — fine for terrain without extreme side slopes.

# 6.  Structural Components

## 6.1  Arm tubes (round profile)

The arms use round aluminum tube rather than square, mostly because it’s easier to source and easier to bond into the printed joints. To keep the design’s logic — the most-loaded arm is the thickest — the rocker uses a bigger tube than the bogie:

| Tube | Profile | Length (center to center) | Qty |
| --- | --- | --- | --- |
| Rocker segment A (front) | Ø25 × 2 mm | 401 mm | 2 |
| Rocker segment B (rear) | Ø25 × 2 mm | 396 mm | 2 |
| Bogie arm | Ø20 × 2 mm | 223 mm | 4 |

Table 4. Aluminum tubes in the mechanism.

The lengths above are center-to-center between pivots. When cutting, subtract the part that slides into each printed joint (26 mm at Joint A, 30 mm at the chassis fitting, 21 mm at Joint B) to get the real cut lengths used on the bench.

## 6.2  3D-printed joints

The intersections of the mechanism aren’t bent tubes or bolted plates — they’re PETG printed parts. Each joint has cylindrical sockets where a tube press-fits and gets pinned with M5 bolts, plus a central boss the pivot axle passes through. The joints are what set the mechanism’s angles, which is exactly why the tubes can be cut dead straight.

| Joint | Sockets | Socket Ø | Angle between sockets | Boss |
| --- | --- | --- | --- | --- |
| Joint A (rocker apex) | 2 | Ø25 | 160° | Ø12 |
| Joint B (bogie pivot) | 2 | Ø20 | 150° | Ø12 |

Table 5. 3D-printed PETG joints.

Suggested print settings: PETG, 5 perimeters, 60% infill, no supports (print with the socket facing up). Each tube is held by 2 M5 bolts per socket. The Ø12 differential axle runs through the central boss, with a printed plain bushing between axle and boss, greased.

## 6.3  Differential axle and chassis connection

The differential axle is a Ø12 × 600 mm steel rod — 600 mm being the chassis width. It’s a passive axle: it stays fixed to the chassis while the rocker bosses turn on it. It connects to the chassis through printed PETG bearing blocks that clamp the rod to the aluminum chassis profile with M4 screws, and the rod is locked with set screws so it can’t spin. When one rocker goes up, the other goes down by the same amount through this rod, keeping the chassis level.

## 6.4  Wheel mounts and steering

The wheels don’t attach straight to the tubes — they sit on intermediate mounts:

- The 2 front and 2 rear wheels (the corners) use C-shaped mounts with an MG995 servo on top. The servo turns on a vertical axis and steers the wheel.

- The 2 middle wheels use L-shaped mounts that are fixed — no servo — just holding the wheel upright and level.

To keep each wheel vertical even though the tube is tilted, the socket on each mount is angled to cancel out the tube’s tilt. Measured from perpendicular to the tube (90°), close it by:

| Mount | Tube | Tube tilt | Rotation from perpendicular |
| --- | --- | --- | --- |
| Front C | Bogie | 15° | −15° (ends at 75° to the tube) |
| Middle L | Bogie | 15° | −15° (ends at 75° to the tube) |
| Rear C | Rocker seg. B | 10° | −10° (ends at 80° to the tube) |

Table 6. Socket orientation for each wheel mount.

Rule of thumb: orient the socket so the mount body hangs plumb (vertical, checked with a level) when the rover sits on flat ground. Then the servo axis is vertical and the wheel sits straight.

## 6.5  Leveling the three wheels

For all three wheels on a side to touch the ground at the same height, the three mounts need the same vertical distance H from the socket (where they grab the tube) down to the wheel center. H is set to 90 mm, enough room to fit the MG995 servo and its bracket above the corner wheels. The middle wheel uses the same H even though it has no servo. With H identical on all three mounts, the wheels end up level automatically.

## 6.6  Materials

| Component | Material |
| --- | --- |
| Rocker and bogie arms | Round aluminum tube |
| Joints and bushings | Printed PETG (5 perimeters, 60% infill) |
| Differential axle and pins | Steel (rod and Ø12 pins) |
| Chassis bearing blocks | Printed PETG |
| Wheel mounts | Printed PETG |
| Steering servos | MG995 (×4) |
| Wheel rim | Printed PETG, two halves |
| Tire | TPU, chevron tread |

Table 7. Material by component.

# 7.  Dimension Summary

| Parameter | Value |
| --- | --- |
| Body width | 600 mm |
| Body length | 1200 mm |
| Wheelbase | 1000 mm |
| Wheel diameter | 200 mm |
| Wheel rpm at 150 mm/s | 14.32 rpm |
| Rocker tube | Ø25 × 2 mm (segments 401 + 396 mm) |
| Bogie tube | Ø20 × 2 mm (223 mm × 4) |
| Joint A angle (rocker) | 160° |
| Joint B angle (bogie) | 150° |
| Differential axle | Steel Ø12 × 600 mm |
| Wheel-mount height (H) | 90 mm (same on all 3) |
| Ground clearance (with mounts) | ≈ 248 mm |
| Obstacle target | 300 mm (1.5 × D) |
| Steering servos | 4 × MG995 (corner wheels) |
| Estimated total mass | 22 kg |

Table 8. Main dimensions (both sides identical).

# 8.  Wheel and Tire

The wheel pairs a printed PETG rim with a TPU 95A tire that has a chevron tread — a low-cost stand-in for the elastic rubber tires that the traction analysis recommends (Singh & Jain, 2020).

| Parameter | Value |
| --- | --- |
| Outer diameter | 200 mm |
| Rim drum diameter | 170 mm |
| Tread width | 80 mm |
| TPU casing thickness | 6 mm |
| Lug height | 9 mm |
| Lug count | 16 (chevron, every 22.5°) |
| Rim material | PETG (two halves, 6× M3) |
| Tire material | TPU (one piece, bonded to rim) |

Table 9. Wheel specification.

# 9.  Test Prototype

Before committing to aluminum, the plan is to build a first test version with plastic tube instead of aluminum for the arms. The point is to validate the geometry and get the motors running without spending much: if something’s wrong with the design, it’s far better to find out on a cheap plastic prototype than after cutting expensive aluminum. The near-term goals are finishing the printable joints, throwing together a low-cost plastic test frame, and getting the drive motors spinning to confirm the whole thing moves the way the Fusion assembly says it should.

# 12.  Conclusions

This report lays out the full rocker-bogie geometry for a 600 × 1200 mm rover that should cruise at 150 mm/s and clear 300 mm obstacles. The 200 mm wheel needs 14.32 rpm, well inside the range of JGB37-520 gearmotors. The centered bogie pivot (Case 2) splits the load 0.30/0.30/0.40, leaving the rear wheel as the most loaded and the rocker as the critical part. The arms are round tube (Ø25 rocker, Ø20 bogie) joined by 3D-printed fittings that set the mechanism’s angles (Joint A at 160°, Joint B at 150°). The differential axle is Ø12 and bolts to the chassis through printed bearing blocks. The four corner wheels steer with MG995 servos, and the three mounts per side share a single 90 mm height so the wheels sit level. Structural verification in Fusion 360 (Section 9) should be finished before any fabrication, with extra attention on the Ø12 axle and the printed joints.

# References

Bickler, D. B. (1998). Roving over Mars. Mechanical Engineering, 120(4), 74–77.

NASA Jet Propulsion Laboratory. (n.d.). Open source rover: Mechanical design documentation. https://open-source-rover.readthedocs.io

Singh, A., & Jain, P. K. (2020). A study on rocker-bogie suspension for a planetary rover prototype. International Journal of Advance Research and Innovation, 8(3), 27–39.

Verma, A., Yadav, C., Singh, B., Gupta, A., Mishra, J., & Saxena, A. (2017). Design of rocker-bogie mechanism. International Journal of Innovative Science and Research Technology, 2(5), 312–338.